Version 2.1 Documentation - May 1998

New in P8 Version 2.1:

Maximum problem dimensions increased:

Max Number of Watersheds = 192 (vs. 24 in Version 2.0)

Max Number of Devices =     48 (vs. 24 in Version 2.0)

Added procedures for importing or exporting watershed data in ASCII:

'Case Read Watershed' - reads watershed data from an ASCII file

'Case Save Watershed' - saves watershed data to an ASCII file

These procedures facilitate data entry in large problems when watershed

data are derived from another application (GIS, spreadsheet, etc.)

P8 Version 2.1 modifications performed with support from CH2MHill, Inc.

The expanded watershed index is accessed via three procedures:

'Case Edit Watershed Index'   (Watershed No.'s 1-64)

'Case Edit Watershed 2Index'  (Watershed No.'s 65-128)

'Case Edit Watershed 3Index'  (Watershed No.'2 129-192)

Save or Read Watershed Data in ASCII Format (Advanced User Mode)

'Case Save Watershed' - saves watershed values to an ASCII file

'Case Read Watershed' - reads watershed values from an ASCII file

These routines allow importing and exporting of watershed input data

from or to other applications (GIS, Spreadsheets, etc.)

File format is illustrated in file 'TEST.TXT', which was created

from Lotus-123 workbook 'TEST.WK4' using the File SaveAs *.TXT option.

Format:

First Line = Case File Name  (for reference, not accessed by program)

Second Line = Field Identifiers   (")

Remaining Lines = Watershed Data

1 line/watershed,  Free Format

Watershed label enclosed in 'quotes'

Fields in same order as 'Case Edit Watershed' input screen

Record Numbers can be in any order (Range 1 - 192)

Any records in an imported watershed file replace those in the current

file, based upon watershed number. Any records in the current file

which are not in the imported file remain intact.

'TEST.CAS' is a sample problem with 1 one watershed of 192 acres.

'TEST2.CAS' is the same problem with 192 watersheds of 1 acre each.

Results should be the same.

'TEST2.CAS' was created from 'TEST.CAS' by reading watershed data

from file 'TEST.TXT' using the 'Case Read Watershed' procedure.

*